### Answers for Week 13 Homework			For the quiz on Friday, May 8

1. covalent

2. silicon is a semiconductor so will conduct electricity if a voltage is applied

silicon dioxide is an insulator so will not conduct electricity

3. Conductors have no gap between their valence and conduction bands.  
Semiconductors have a small band gap, and insulators have a large one.

4. Si or Ge:  4		GaAs: 3, 5		CdTe: 2, 6

5. zinc blende

6. Dope it with elements containing a different number of valence electrons.

7. n-type:  N, P, As, Sb, Se, Te		p-type:  B, Al, Ga, In, Zn, Cd

8.  color changes (to blue), conductivity increases

9.  Diodes force current to travel only one direction (from n to p doped regions).
Transistors may be used to amplify electrical signals.

10.  chemical vapor deposition (CVD), molecular beam epitaxy, etching, photolithography

11.  Any impurities in a semiconductor greatly affect its conductivity.

| 12. | LED | incandescent bulb |
| --- | --- | --- |
| wavelength | one color | broad spectrum |
| electricity to light conversion | efficient | inefficient (mostly heat produced) |
| response to turning on current | fast creation of light | must heat up before producing light |
| longevity | solid crystal, long life | delicate filament wears out |

13. earth abundant elements: copper zinc tin sulfoselenide, lead sulfide, iron sulfide, tin sulfide

Perovskites are hybrid organic-inorganic materials that are unfortunately not very stable.

14. Recommendations for manufactures, and the corresponding Principles of Green Engineering: design devices for durability (#7), reuse, and recycling (#3, #9, #11);

eliminate hazardous materials from designs (#1); use scrap/recycled materials (#12);

make devices reparable (#6); buy back or otherwise encourage return of used devices.

15. will be discussed in class on Friday